# Sonic Firmware Documentation

RS232 version uses the kenwood cable and has 200 history entries

USB version uses USB-C cable and has 100 history entries,

◀️  Join the SonicFw Telegram https://t.me/SonicFw

📺 Youtube Channel https://www.youtube.com/@robby_69400

💾 DOWNLOAD select RAW file DOWNLOAD_HERE

🙏 Many thanks to Sonic Team : Zylka, Kolyan, Iggy, Toni, Yves and Francois.

This software is a fork of Armel F4HWN firmware with code from NTOIVOLA, EGZUMER and DUAL TACHYON.

# Installation

use the same procedure as F4HWN, after that :

- If necessary, you can do a full reset of the radio. press the PTT and Fn2 buttons  , turn on the radio. A warning message will be displayed on the screen. hold the buttons for 4 seconds before resetting.

## CHIRP Driver

Chirp_Sonic.py in DOWNLOAD_HERE

This driver also allows

- customization of the 50 bands on channels 976 to 1024.

Select start frequency  and offset as stop frequency, step and modulation

## EXPLANATION OF BUTTON SYMBOLS:

1 – Simple press.

L1 – Long press.

F1– Press after the F button.

## MR MAIN SCREEN (CHANNELS) | VFO (FREQUENCY)

L1 – (MR) – Transfer the channel frequency to VFO mode. (VFO) – Change of range.

L2 – (MR and VFO) – VFO change (in MAIN ONLY mode, icon indication).

L3 – Change the MR/VFO modes on the selected channel.

L4 – (MR and VFO) – Bandwidth change.

L5 – (MR) – Change the list number for the channel. (VFO) – Change of step.

L6 – (MR & VFO) – Power Change (L/M/H).

L7 – (MR and VFO) – On/Off flashlight indication when receiving a signal.

L9 – (MR and VFO) – On/Off constant backlight.

L0 – AM/FM/USB Modulation Change

F1 – 1CALL – Quick Call.

F2 – (MR) – Delete Channel. (VFO) – Preserve the frequency.

F7 – Spectrum analyzer in scanlist mode.

F8 – Band Spectrum Analyzer.

F9 – Spectrum analyzer in frequency mode.

F0 – Radio broadcasting (FM radio).

Fn1 and Fn2—Side assignable buttons.

<= and => – Navigation buttons.

MA – Menu.

EXIT – Exit.

#F – Functional "F".

## FREQUENCY OR LIST SCANNING IN MR/VFO MODES

In MR (Channel) and VFO (frequency) modes, you can scan by channel or by frequency range with a set step. To scan in frequency mode, press  the * button,  this will start the scan. In channel mode, the * button  changes the scan lists. You can also change the lists by simply entering numbers from the keyboard.

## OPERATION IN SPECTRUM MODES

Different types of spectrum are called by the quick buttons F7 , F8, F9 in any of the spectrum modes, you can change them by pressing 6.

1 – Skip the frequency in the scan mode.

2 – Turn on/off the backlight).

3 – Bandwidth in the spectrum. In the options menu, change the value.

4 – List of scanned lists or bands. Already in the selection mode, mark the selected ones with it (highlighted in bold and with a checkmark on the right).

5 – Options menu. Navigation through the menu <= and =>. Change the values of buttons 1 and 3.

6 – Changing the modes of the spectrum analyzer Scanlists/Frequency/Range/Bands.

7 - Save Settings. Save after configuring and selecting the lists and the options will be applied once enabled.

8 – Change the display screen.

9 – Change of AM/FM/USB modulation.

0 – Switch to the history mode.

* and #  DYNSQL Level setting  - RSSI spike threshold  for instant peak capture. For weak and fuzzy signals, increase or disable.

L* and L# DYNSQL OFF This will fill the history, but won’t stop.

## Keys in history

1 – Clears all history (use 8 to save).

2 – Saves the frequency to a free channel.

3 – Delete the entry.

4 – Clears all black listed entries (use 8 to save).

5 – Scans history entries.

7 - Clears all non black listed entries (use 8 to save).

8 –Saves the history.

0 –Sort the history.

## PARAMETERS OF THE SPECTRUM ANALYZER

DELAYRSSI – Delay  in RSSI measurement  before frequency change (in ms) – affects the scanning speed.

SPECTRUMDELAY – Pause time after signal detection (in ms) – if 0, immediately resumes scanning.

MAXLISTENTIME – Maximum time to listen to the signal (in seconds) – after that, the scan continues.

FSTART – Initial frequency of the scan range (in MHz).

FSTOP – End frequency of the scan range (in MHz).

STEP – Frequency scan step (in kHz): 0.01; 0.05; 0.1; 0.5; 1.0; 2.0; 5.0; 6.25; 10.0; 12.5; 20.0; 25.0; 50.0; 100.0.

LISTENBW – Bandwidth: Wide (25 kHz) / Narrow (12.5 kHz)/ AIR (8.33kHz).

MODULATION – Modulation type: FM/AM/USB.

RX_BACKLIGHT_ON – Backlight when receiving a signal: ON/OFF.

POWERSAVE – Power saving mode: Off /200ms /500ms /1s /2s /5s.

NOISLVL_OFF – Noise level for disabling reception (above this – the signal is counted).

POPUPS – Popup display time (in ms) – 0 = disabled.

RECORD TRIG – An additional threshold for recording in history. The higher the threshold, the stronger the signal will be recorded.

KEY UNLOCKED – THE TIME BEFORE THE KEYPAD IS LOCKED.

GLITCHMAX – The threshold  of the GLITCH indicator BK4829. Increase – more buggy signals will be considered valid.

SOUND BOOST – Amplify sound. Increase bass frequencies.

PTT – What to do when pressing PTT: LAST VFO /NINJA /LAST RECEIVED.

NINJA chooses a random frequency in the current scanlist.

MONITOR SL – Includes the "MON" priority list in all scan lists.

RESET DEFAULT – Reset all spectrum parameters to factory settings.

Light OR Advanced – Reset all spectrum parameters to factory settings.

## DETAILED MONITORING IN SPECTRUM MODE VIA THE "MENU" BUTTON

Move through the low-level register menu with buttons 2 and 8. Change values with the <= and => buttons

LNAS – Low Noise Amplifier (LNA) – signal amplification stages in front of the mixer.

LNA – Gain of the first LNA stage – higher = stronger signal but more interference.

PGA – Programmable Gain Amplifier (after LNA). – Balance between sensitivity and distortion.

MIX – Mixer Gain (Frequency Mixing) – Higher = Better Dynamic Range but More Noise.

XTAL F MODE SELECT – Quartz filter (XTAL) operating mode. Filtering (reduces noise, but can distort).

OFF AF RX DE-EMP – Disable demphasis (pre-distortion correction) during reception.

GAIN AFTER FM DEMOD – Adjusts the reception volume.

RF TX DEVIATION – Transmission frequency deviation (Fm deviation) – affects the transmission bandwidth (higher = louder).

COMPRESS AF TX RATIO – Compression ratio of audio during transmission – compresses the dynamic range.

COMPRESS AF TX 0 DB – Compression level 0 db during transmission – the compression start point.

COMPRESS AF TX NOISE – Noise reduction in quiet areas.

MIC AGC DISABLE – Disable automatic microphone gain control (AGC).

AFC RANGE SELECT – Operating range of AFC (Automatic Frequency Tuning).

AFC DISABLE – Disable AFC.

AFC SPEED – AFC Speed.

## OPERATION IN BROADCAST RADIO MODE "FM"

The firmware has a simplified FM radio. Use the <= and => buttons  to search for the nearest available broadcasting frequency in automatic mode.  The * button  will change the Auto mode  to manual search.

Press key 1 to 6 to select the corresponding memory.

Long press key 1 to 6 to save a frequency to that memory.

## MAIN MENU

THERE IS NO HIDDEN MENU IN THE FIRMWARE

|  | Step | The frequency step (in kHz), UP and Down buttons change the frequency to this value, and you can only set the frequency to half of this value. For CB - 5/10 kHz; SatCom, Avia - 5/25 kHz; The rest is 6.25/12.5 kHz. |
| --- | --- | --- |
|  | SQL | The level of noise cancellation (Squelch) is from 0 (always open) to 9 (very strict). Set for both bands (A and B) |
|  | Mode | Demodulation mode, default - FM, AM and USB can only be used for listening |
|  | RxMode | Sets how the high and low frequencies are used on the screen (MAIN ONLY - Always transmits and listens on the main frequency (MO), DUAL RX RESPOND - Listens to both frequencies, if the signal is received on the secondary frequency, it is fixed on it for a couple of seconds so that you can answer the call (DWR), CROSS BAND - always transmits on the primary frequency and listens on the secondary frequency (XB), MAIN TX DUAL RX - always transmits to the main, listens to both (DW). |
|  | RxDCS | Setting Digital Coded Squelch, if enabled, the squelch will only open after receiving that subtone, applicable for FM modulation only. |
|  | RxCTCS | Setting an analog subtone to reception (Continuous Tone Coded Squelch), if the function is enabled, the noise canceler will open only after receiving this subtone, applicable only for FM modulation. |
|  | TxDCS | Setting the digital encoded digital subtone to transmit, if the function is enabled, the transceiver will transmit this tone, applicable only for FM modulation. |
|  | TxCTCS | Setting the analog subtone to transmit, if the function is enabled, the transceiver will transmit this tone, applicable for FM modulation only. |
|  | TxOffs | Transmitter frequency offset value in MHz, transmit/receive at different frequencies. |
|  | TxODir | Direction (+/-) of transmitter frequency offset when transmitting/receiving at different frequencies |
|  | W/N | The bandwidth used by the transceiver (WIDE (25 kHz), NARROW (12.5 kHz). |
|  | SetNFM | Sets Narrow FM to Narrow or Narrower |
|  | Compnd | Compander - Soft audio filter for clearer sound. Amplifies the received desired signal even when the signal is weak at the transmitting microphone, improving the signal-to-noise ratio. To work, it must be turned on on both walkie-talkies. |
|  | Roger | On/off "roger beep" (end-of-pass tone). |
|  | 1 Call | One key call channel, allows you to quickly switch to a channel by pressing the 9-Call button |
|  | STE | Noise suppressor, eliminates noise at the end of the transmission. When PTT is released, a short 55Hz tone is emitted, signaling to other transceivers to mute |
|  | RP STE | Noise Cancelling Tail Eliminator Repeater |
|  | ChName | Name of the memory channel. |
|  | ChDisp | Choose how the saved channel will be displayed on the walkie-talkie display |
|  | F1Shrt | Function assignment by short pressing the F1 side key. |
|  | F1Long | Function assignment when the F1 side key is pressed for a long time. |
|  | F2Shrt | Function assignment by short pressing the F2 side key. |
|  | F2Long | Assign a function when you press the F2 side key for a long time. |
|  | M Long | Assign a function when you long press the M key |
|  | KeyLck | Automatically lock the keypad after a set time |
|  | TxOut | Transmit Timeout - The amount of time that PTT can be held during transmission (in seconds). Protection against accidental clamping of PTT. |
|  | BatSav | Battery Save: OFF / 1:1 / 1:2 / 1:4 / 1:8. |
|  | Mic | Microphone sensitivity (microphone amplifier). |
|  | POnMsg | Enable/disable the display of a message when turned on (Power On Message). |
|  | BLTime | Screen backlight time when buttons are pressed. |
|  | BLMin | Minimum backlight brightness (in percentage). |
|  | BLMax | Maximum backlight brightness (in percentage). |
|  | BLTxRx | Transmit/Receive Backlight: OFF/TX/RX/TX+RX. |
|  | SetInv | Enable walkie-talkie screen color inversion |
|  | BtnInv | Reverse the direction of the UP and DOWN buttons |
|  | SetLck | Setting the keypad lock type with or without PTT. |
|  | BatTxt | Battery Display Type, %, V or disabled. |
|  | MicBar | Enabling/disabling the microphone bar |
|  | SetTmr | Set the display of RX/TX timers on the screen |
|  | F Lock | Setting up Frequency Lock. What frequencies are allowed or prohibited to work on. |
|  | BatCal | Battery Calibration. |
|  | BatTyp | Choice of battery type (Li-ion, NiMH, etc.) and capacity. |
|  | Reset | Channel Preservation Reset (NOCH) or Full Factory Reset including Saved Channels (ALL) |

## Chirp Configuration

- Memory channels 975 to 1024 store custom band settings. For each, define:

  - Band Name.

  - Start and End frequency (via offset).

  - Modulation & Frequency Step.

- Scanlist Monitor setting Select up to 20ch to Monitor scanlist anywhere in the channels.