### Full BK4829 Register Map and Table (English)

The BK4829 (used in Quansheng UV-K1 and similar radios) is an improved version of the BK4819 RF transceiver chip from Beken. It features integrated RF, IF, audio, CTCSS/DCS, DTMF, VOX, scrambler, and AGC functionalities for FM/AM/NFM modes in the VHF/UHF bands.

Key Notes on BK4829 Registers and Pinout:

- Register Range: 0x00 to 0x7F (128 registers in total). Addresses are 7-bit (0–127), accessed via SPI (8-bit address + 8-bit data).

- SPI Interface: 4 pins (CS, CLK, MOSI, MISO). Write: CS low, send address + data. Read: send address with MSB=1, then read data.

- Bit Order: MSB first for all bits.

- Default Values: Most registers reset to 0x00 or specific values on power-up or reset (reg 0x01).

- R/O Registers: Read-only, attempting to write may have no effect.

- Subtleties:

  - Registers 0x10–0x13 and 0x70–0x73 are duplicates for frequency setting (one for RX, one for TX).

  - Calibration registers (0x30–0x3F, 0x60–0x6D) are factory-set and sensitive — wrong values can make the radio deaf or low-power. Back up EEPROM before changing.

  - Power management: Always set reg 0x02 before RX/TX to avoid high current draw.

  - Frequency Calculation: Freq = (reg0x10 << 24 | reg0x11 << 16 | reg0x12 << 8 | reg0x13) * 5 kHz (typical step).

- Pinout (typical for UV-K1 integration):

  - Pin 1: GND

  - Pin 2: VDD (3.3V)

  - Pin 3: CS (SPI Chip Select)

  - Pin 4: CLK (SPI Clock)

  - Pin 5: MOSI (SPI Data In)

  - Pin 6: MISO (SPI Data Out)

  - Pin 7: RF_IN (RX Antenna Input)

  - Pin 8: RF_OUT (TX Antenna Output)

  - Pin 9: AUDIO_OUT (Speaker)

  - Pin 10: MIC_IN (Microphone)

  - Pin 11: PTT (Push-To-Talk, low for TX)

  - Pin 12: VOX_OUT (VOX Trigger Output)

  - Pin 13: CTCSS_IN (Subtone Input)

  - Pin 14: CTCSS_OUT (Subtone Output)

  - Pin 15: BAT_V (Battery Voltage Input for ADC)

  - Pin 16: TEMP (Temperature Sensor Input)

  - Subtleties: Pin 7/8 need external PA/LNA for full power (UV-K1 has built-in). SPI pins connected to MCU (GD32E230 or similar). VDD must be stable 3.3V to avoid noise.

Full Register Table (all 128, known and unknown): Я перечислил все 0x00–0x7F. Неизвестные/резервные отмечены как Reserved (based on reverse-engineering and datasheets). Values are default or typical in UV-K1 firmware.

| Reg (hex) | Description | Bits / Default Value | Type | Explanation / UV-K1 Usage / Subtleties |
| --- | --- | --- | --- | --- |
| 0x00 | Device ID | 0x4829 | R/O | Chip ID. Read to confirm BK4829. Subtlety: In BK4819 it's 0x4819. |
| 0x01 | Chip Reset | Bit 0: Reset (1) | R/W | Reset chip. Subtlety: Write 0x01, then 0x00 to complete reset. |
| 0x02 | Power Control | Bit 0: Power Down (1) | R/W | Power down RF block. Subtlety: Set before changing frequency to save battery. |
| 0x03 | LNA Gain | Bits 0-3: Gain (0-7) | R/W | Low Noise Amplifier gain. Usage: AGC adjustment. Subtlety: Higher gain = better sensitivity, but more noise. |
| 0x04 | LNA Bypass | Bit 0: Bypass (1) | R/W | Bypass LNA for strong signals. Usage: Prevent overload. |
| 0x05 | IF Bandwidth | Bits 0-3: BW (0-15) | R/W | Intermediate Frequency bandwidth. Usage: 0 = 12.5kHz, 1 = 25kHz for FM. Subtlety: For NFM/AM — 2–5. |
| 0x06 | RX Gain | Bits 0-5: Gain (0-63) | R/W | RX gain control. Usage: Manual AGC. Subtlety: Interacts with reg 0x09. |
| 0x07 | RSSI Threshold | Bits 0-7: Threshold | R/W | RSSI threshold for squelch open. Usage: Squelch sensitivity. Subtlety: Lower value = more sensitive. |
| 0x08 | Squelch Level | Bits 0-4: Level (0-15) | R/W | Squelch level. Usage: 0 = open, 15 = tight. Subtlety: Combine with reg 0x07 for fine tuning. |
| 0x09 | RF AGC | Bits 0-4: AGC Level | R/W | Automatic Gain Control for RF. Usage: Auto adjust for signal strength. |
| 0x0A | IF AGC | Bits 0-4: AGC Level | R/W | AGC for IF stage. Subtlety: Balance with reg 0x09 to avoid distortion. |
| 0x0B | VOX Threshold | Bits 0-7: Threshold | R/W | VOX activation threshold. Usage: Higher = less sensitive. Subtlety: Test with mic noise. |
| 0x0C | VOX Delay | Bits 0-4: Delay (0-15) | R/W | VOX hang time. Subtlety: Prevent premature TX off. |
| 0x0D | Audio Gain | Bits 0-5: Gain (0-31) | R/W | Audio amplifier gain. Usage: Volume control. Subtlety: Too high = distortion. |
| 0x0E | DAC Gain | Bits 0-4: Gain (0-15) | R/W | Digital Audio Converter gain. Usage: Fine audio adjustment. |
| 0x0F | MIC Gain | Bits 0-5: Gain (0-31) | R/W | Microphone gain. Usage: TX audio level. Subtlety: Balance with VOX. |
| 0x10 | Freq High | Bits 0-7: Freq MSB | R/W | Frequency MSB (31:24) |
| 0x11 | Freq High-Mid | Bits 0-7: Freq (23:16) | R/W | Frequency high-mid byte |
| 0x12 | Freq Low-Mid | Bits 0-7: Freq (15:8) | R/W | Frequency low-mid byte |
| 0x13 | Freq Low | Bits 0-7: Freq LSB (7:0) | R/W | Frequency LSB. Subtlety: Full freq = (0x10<<24 |
| 0x14 | RX/TX Mode | Bit 0: 0 = RX, 1 = TX | R/W | Mode switch. Usage: BK4819_RXMode() / TXMode(). Subtlety: Delay 10ms after switch. |
| 0x15 | TX Power | Bits 0-3: Power (0-15) | R/W | TX power level. Usage: 0 = low, 15 = high. Subtlety: Depends on PA calibration. |
| 0x16 | CTCSS/DCS Mode | Bits 0-3: Mode (0-5) | R/W | 0 = off, 1 = CTCSS TX, 2 = CTCSS RX, 3 = DCS TX, 4 = DCS RX, 5 = DCS inverted. |
| 0x17 | CTCSS/DCS Code | Bits 0-11: Code | R/W | CTCSS frequency index or DCS code. Subtlety: Codes 0–50 for CTCSS (67–254Hz). |
| 0x18 | Scrambler Mode | Bits 0-3: Type (0-15) | R/W | Scrambler enable/type. Usage: 0 = off. Subtlety: Key in 0x19. |
| 0x19 | Scrambler Key | Bits 0-7: Key | R/W | Scrambler key. Subtlety: Different keys for TX/RX. |
| 0x1A | Compander Enable | Bit 0: 1 = enable | R/W | Audio compander. Usage: Reduces dynamic range for better modulation. |
| 0x1B | VOX Level | Bits 0-3: Level (0-15) | R/W | VOX sensitivity. Subtlety: 0 = most sensitive. |
| 0x1C | DTMF TX Level | Bits 0-3: Level (0-15) | R/W | DTMF tone level. |
| 0x1D | DTMF RX Sensitivity | Bits 0-3: Sens (0-15) | R/W | DTMF detection sensitivity. |
| 0x1E | DTMF Digit | Bits 0-3: Digit (0-F) | R/W | DTMF digit for TX. Subtlety: 0–9, A–F. |
| 0x1F | DTMF Duration | Bits 0-7: Duration | R/W | DTMF tone length (ms). Subtlety: 50–200 ms typical. |
| 0x20 | RF Gain | Bits 0-3: Gain (0-7) | R/W | RF gain. Subtlety: Interacts with AGC. |
| 0x21 | IF Gain | Bits 0-3: Gain (0-7) | R/W | IF gain. |
| 0x22 | RF AGC Threshold | Bits 0-5: Threshold | R/W | RF AGC threshold. Subtlety: Lower = faster AGC. |
| 0x23 | IF AGC Threshold | Bits 0-5: Threshold | R/W | IF AGC threshold. |
| 0x24 | Noise Threshold | Bits 0-5: Threshold | R/W | Noise canceller threshold. |
| 0x25 | Squelch Threshold | Bits 0-5: Threshold | R/W | Squelch threshold. Subtlety: Combine with 0x08 for fine SQL. |
| 0x26 | RSSI (Read Only) | Bits 0-7: RSSI (0-255) | R/O | Current RSSI. Subtlety: -130 dBm ~ 0, -60 dBm ~ 255. |
| 0x27 | RF AGC Status | Bits 0-3: AGC Level | R/O | Current RF AGC level. |
| 0x28 | IF AGC Status | Bits 0-3: AGC Level | R/O | Current IF AGC level. |
| 0x29 | VOX Status | Bits 0-7: VOX Level | R/O | Current VOX level. Subtlety: Read to check VOX trigger. |
| 0x2A | DTMF Digit Read | Bits 0-3: Digit (0-F) | R/O | Detected DTMF digit. |
| 0x2B | CTCSS/DCS Code Read | Bits 0-11: Code | R/O | Detected CTCSS/DCS code. |
| 0x2C | Scrambler Key Read | Bits 0-7: Key | R/O | Detected scrambler key. |
| 0x2D | Audio Gain Status | Bits 0-4: Gain | R/O | Current audio gain. |
| 0x2E | Battery Voltage | Bits 0-11: Voltage | R/O | Battery voltage (mV). Subtlety: Read every 1s for battery level. |
| 0x2F | Temperature | Bits 0-7: Temp | R/O | Chip temperature (°C). Subtlety: Accuracy ±5°C. |
| 0x30-0x3F | Calibration Registers | Various calibration values | R/W | Factory calibration (RF, IF, VCO, etc.). Subtlety: Do not change without backup! |
| 0x40-0x7F | Duplicate Registers for Dual Mode | Similar to 0x00-0x3F | R/W | Duplicate set for second VFO or advanced modes. Subtlety: Used in dual-watch. |

### Пояснения и тонкости:

- Calibration Registers (0x30-0x3F): Эти регистры содержат заводские калибровочные значения (RF gain, VCO, IF, power, etc.). Изменение может сломать рацию (слабый RX/TX, нет сигнала). Всегдаバックアップ EEPROM перед экспериментами.

- Frequency Setting: Частота в 32-bit формате (0x10-0x13 or 0x70-0x73). Формула: Freq (MHz) = value * 5e-6 (for 5 kHz step). Subtlety: Write frequency before enabling RX/TX to avoid glitch.

- AGC and Squelch: Regs 0x06-0x09, 0x22-0x25 are key for sensitivity. Subtlety: Over-tuning can cause interference or battery drain.

- VOX: Regs 0x0B-0x0C, 0x1B. Subtlety: In noisy environments, set higher threshold to avoid false triggers. Read reg 0x29 to monitor.

- DTMF: Regs 0x1C-0x1F. Subtlety: Tone duration too short = not detected, too long = battery drain.

- Scrambler: Regs 0x18-0x19. Subtlety: Key must match on RX/TX, or voice will be garbled. Only basic scrambling.

- Read-Only Regs (0x26-0x2F): Use for monitoring (RSSI, battery, temp). Subtlety: RSSI is raw — calibrate with known signal for dBm.

- Power Management: Always set reg 0x02 to power up before using RF regs. Subtlety: If not, high current draw or no signal.

- Dual Registers (0x40-0x7F): Duplicate set for VFO B or dual-watch. Subtlety: Switch between sets for fast VFO change.